FM / FSK Transmitter Module TXF2000U / FSK
The TXF*2000U UHF band high power FM transmitter is an ideal design module used for Paging system, Data link system, Security System, and Remote data system, etc..

Main Function

• Complete with 1 crystal and 1 SAW Filter design

• Frequency Band: 430~450,450~470,860~880,915~935MHz

• FM or direct FM modulation from 2K to 30K, typ ± 4.5K.

• Data rate up to 6.4Kbps, typ 512bps/1200bps/2400bps.

• High Power output up to 1.5 Watt

• Wide operate Volt from DC10V~24V, typ DC12V 0.5A(Max).

• Wide operate temperature from -20℃ -- +70℃
• Small size

[image: image1.png]



Specification

Frequency stability 10ppm at 25℃ (5ppm for option)

Operation frequency 430~450,450~470,860~880,915~935MHz.

Data Rate Up to 6.4Kbps by 2 level.

Modulation ±2K~30K adjustable, type ± 4.5K for narrow band FM/FSK  
Channel spacing 12.5K or 25KHz

Working Voltage DC10-24V, type DC12V

RF power output Max 1.5W. 22% duty.

Power consumption Standby 5mA, TX ON 450mA.(at DC12V 1.5 watt output)

Working temperature range -20℃-+70℃.

Module Size 77mm X 27mm X 15mm.

Weight 18 grams.

Pin Number Name Description

1- Data Data input

2 -GND System GND

3 -V+ Power Supply DC V+ 10V to 24V

4 -PTT Low for TX active

Pin Lead size: 2.54mm 4 pin single

Can be use with RMF*600U or RMB*500XS in same frequency.

[image: image2.png]



